Your question seems to describe placing two telescopes (I'll assume that's what "telescops" means) separated by about 2 km in space, aligned one "on top" of the other, and using the second as a magnifier for the first to enhance observations. If interpreted literally as a single extended telescope (like a very long refractor or reflector with the optics separated), the setup wouldn't inherently improve resolution—it would still be limited by the diameter of the primary aperture (diffraction limit), though it could provide higher angular magnification depending on the focal lengths. However, that wouldn't let you "see more" in terms of finer details; it'd just make the image larger but potentially blurrier if not matched properly.

A more powerful interpretation, especially for space-based astronomy, is treating this as an **optical interferometer**: combining the light from two separate telescopes separated by a baseline (the 2 km distance) to act as a virtual telescope with an effective aperture equal to that separation. This technique synthesizes much higher angular resolution than a single telescope of similar mirror size could achieve. Concepts like NASA's proposed Stellar Imager (shown above) involve formation-flying telescopes in space for exactly this purpose, though typically with more than two elements and variable baselines up to kilometers or more.

### How Interferometry Works Here

- In optical interferometry, the telescopes collect light from the same distant object, and their signals are combined (either optically or electronically via intensity correlations) to measure interference patterns.

- The key benefit is angular resolution (the smallest detail you can distinguish), given approximately by θ ≈ λ / B, where λ is the observing wavelength and B is the baseline separation.

- For intensity interferometry (a robust method for long baselines, as it doesn't require precise optical path alignment and is less affected by vibrations in space), this enables imaging at scales impossible with single apertures.

### Calculation of Improvement

Assuming visible light (λ = 550 nm, typical for optical astronomy) and a 2 km baseline:

- Interferometer resolution: ≈ 5.67 × 10^{-5} arcseconds (or about 57 microarcseconds).

To explain the math step-by-step for this closed-ended calculation:

1. θ (in radians) ≈ λ / B = (550 × 10^{-9} m) / 2000 m = 2.75 × 10^{-10} radians.

2. Convert to arcseconds: Multiply by (180 × 3600 / π) ≈ 206265, yielding 2.75 × 10^{-10} × 206265 ≈ 5.67 × 10^{-5} arcseconds.

- For comparison, the Hubble Space Telescope (2.4 m diameter) has a diffraction-limited resolution of ≈ 0.058 arcseconds at the same wavelength.

To explain:

1. θ (in radians) ≈ 1.22 × λ / D = 1.22 × (550 × 10^{-9} m) / 2.4 m ≈ 2.80 × 10^{-7} radians.

2. Convert to arcseconds: 2.80 × 10^{-7} × 206265 ≈ 0.058 arcseconds.

- **Improvement factor**: Hubble's resolution / Interferometer's resolution ≈ 0.058 / 0.000057 ≈ 1017 times finer detail. In other words, you could resolve structures about 1000 times smaller in angular size.

This means "seeing more" in the sense of unprecedented sharpness: for example, directly imaging surface features on nearby stars (like starspots, granulation, or mass ejections), resolving binary star interactions, or even details in protoplanetary disks around other stars. Ground-based interferometers are limited by atmosphere, but in space, this setup could push to microarcsecond scales with km baselines.

### Practical Notes

- Challenges: Precise formation flying to maintain alignment (within fractions of a wavelength), data correlation, and light collection (since interferometers have less total collecting area than a single large mirror, they're better for bright sources or high-contrast imaging).

- Existing/proposed systems: NASA's canceled SPECS mission aimed for km baselines in far-IR for exoplanet studies, while intensity interferometry could work with simpler electronics over such distances.

- If your intent was a literal stacked magnifier (not interferometry), the "more" would be limited to magnification boost (e.g., effective focal length ~2 km, potentially 30–50x Hubble's if similar optics), but without resolution gain, you wouldn't resolve new details—just enlarge the view.

If this isn't what you meant, clarify the telescope sizes or exact setup!

To further improve both angular resolution (the ability to distinguish fine details in terms of angular size) and overall observational detail (such as image quality, sensitivity to faint features, spectral information, or reconstructed imaging) in a space-based two-telescope interferometer setup like the one you described (with a ~2 km baseline), we can build on established astronomical techniques and proposed missions. The core principle remains interferometry, but scaling it up involves engineering challenges like precise formation flying (maintaining alignment via thrusters and lasers) and data processing. Below, I'll outline key ways to enhance both aspects, drawing from current concepts in optical/UV/IR space interferometry.

### 1. Improving Angular Resolution

Resolution is primarily limited by the diffraction limit, approximated as θ ≈ λ / B (in radians), where λ is the wavelength of light and B is the baseline (separation between telescopes). Your 2 km setup already offers ~1000x better resolution than Hubble's 2.4 m single aperture at visible wavelengths (as calculated previously). To push this further:

- **Increase the Baseline Length**: Extend the separation to tens or hundreds of kilometers (or even thousands for ultimate setups). This directly shrinks θ, allowing you to resolve smaller angular features, like finer details on stellar surfaces, exoplanet atmospheres, or protoplanetary disks.

- Example: Scaling to a 100 km baseline (feasible with formation-flying spacecraft at Lagrange points like L2 for stability).

- Calculation of improvement (using visible light, λ = 550 nm = 5.5 × 10^{-7} m):

1. New resolution θ ≈ λ / B = (5.5 × 10^{-7}) / (100,000 m) = 5.5 × 10^{-12} radians.

2. Convert to arcseconds: Multiply by 206,265 (radians to arcseconds factor) ≈ 1.14 × 10^{-6} arcseconds (1.14 microarcseconds).

3. Compared to your 2 km setup (previous θ ≈ 5.67 × 10^{-5} arcseconds): Improvement factor = (5.67 × 10^{-5}) / (1.14 × 10^{-6}) ≈ 50x finer.

4. Relative to Hubble (0.058 arcseconds): ~50,000x finer overall.

- Proposed missions like NASA's Stellar Imager (SI) aim for variable baselines up to 500-1000 km, enabling resolution of starspots or magnetic fields on stars within 100 light-years.

- Even longer: Earth-Moon distance (~384,000 km) baselines have been conceptualized for microarcsecond astrometry, though vibration and alignment become extreme challenges.

- **Shorter Wavelengths**: Observe in UV (λ ~200-300 nm) instead of visible light to halve θ without changing B. This requires UV-optimized mirrors and detectors but reveals hotter stellar phenomena.

- **Advanced Interferometry Modes**: Use chromatic intensity interferometry, which correlates light intensities rather than amplitudes, allowing longer baselines without sub-wavelength alignment precision. This can push resolution by factors of 10-100 beyond basic setups.

### 2. Getting More Observational Details

Higher resolution alone doesn't guarantee "more details" if the signal is noisy or incomplete. To enhance image fidelity, sensitivity, and information depth:

- **Add More Telescopes to the Array**: Instead of just two, use 10-30 smaller telescopes (e.g., 0.5-1 m mirrors each) in a reconfigurable formation. This "fills" the uv-plane (a mathematical space representing spatial frequencies) more completely, enabling full image reconstruction via algorithms like CLEAN or MEM, rather than just fringe patterns.

- Benefit: Turns sparse interferometry into high-fidelity imaging, revealing complex structures like stellar granulation or binary star interactions.

- Light-gathering power scales with the total collecting area (sum of all mirrors), improving signal-to-noise ratio (SNR) for faint details. For example, 20 telescopes with 1 m mirrors each equals the light collection of a ~4.5 m single telescope but with array-level resolution.

- Calculation of SNR improvement (simplified): SNR ∝ √(total area × integration time). Doubling telescopes (same size) doubles area, improving SNR by √2 ≈ 1.41x for the same exposure.

- Examples: The proposed Space Interferometry Mission (SIM) or Terrestrial Planet Finder-Interferometer (TPF-I) use 4-6 collectors for exoplanet spectroscopy; Stellar Imager envisions 20-30 for stellar astrophysics.

- **Larger Individual Apertures**: Upgrade from small mirrors (e.g., 0.3 m in basic setups) to 1-2 m each. This boosts photon collection without needing more units, allowing detection of dimmer features or shorter exposures.

- Trade-off: Heavier spacecraft, but modern deployable optics (like those in JWST) make it viable.

- **Multi-Wavelength and Spectral Capabilities**: Equip telescopes for simultaneous UV, visible, and near-IR observations. This adds "color" information, revealing temperature gradients, chemical compositions, or Doppler shifts for 3D mapping.

- Techniques like spectral imaging interferometry combine high resolution with hyperspectral data, enhancing details in protoplanetary disks or active galactic nuclei.

- **Better Data Processing and Integration**: Use AI-driven reconstruction (e.g., machine learning for uv-plane filling) and longer observation times. Space eliminates atmospheric distortion, so integrate over hours/days for ultra-high SNR.

### Practical Implementation and Challenges

- **Formation Flying**: Use GPS, lasers, and micro-thrusters for nanometer-level positioning. Testbeds like ESA's Proba-3 demonstrate this at ~150 m separations; scale to km for your setup.

- **Orbit Choices**: Place in halo orbits at L2 (like JWST) for thermal stability and minimal drag, or geostationary for Earth-observation variants.

- **Costs and Feasibility**: Start small (your 2 km dual setup ~$500M-$1B); full arrays like SI could cost $5B+ but yield breakthroughs in stellar evolution or exoplanet habitability.

- **Limitations**: Dim sources require long exposures; vibrations from spacecraft must be damped. Ground-based arrays (e.g., VLTI) test concepts but space avoids turbulence.

This could let you "see" details like resolving exomoons or stellar oscillations—far beyond single telescopes. If you specify telescope sizes, wavelengths, or targets, I can refine the math!

If we scale up your original two-telescope setup (with a ~2 km baseline in space) to more advanced configurations—like those proposed in concepts such as NASA's **Stellar Imager (SI)** or similar space-based optical/UV interferometers—the improvement in what we can "see" becomes dramatic. This means resolving much finer angular details on stars and other objects, effectively turning a blurry point into a detailed map of surface features, magnetic activity, or surrounding structures.

### Quick Recap of Your Starting Point (2 km baseline)

- At visible light (~550 nm wavelength): ~57 microarcseconds (μas) angular resolution.

- Hubble (2.4 m single mirror): ~58 milliarcseconds (mas) → your setup is already ~**1000 times sharper** in angular detail.

### Scaling Up: Realistic Advanced Setups

Modern proposals use **variable baselines** (reconfigurable from hundreds of meters to ~0.5–1 km or more) with **20–30 smaller telescopes** (1–2 m mirrors each) in formation flying at stable points like Sun-Earth L2. This synthesizes a much larger effective aperture for imaging, not just fringe measurements.

- **Typical proposed resolution** (from Stellar Imager concepts): ~50–200 μas at UV/optical wavelengths, often targeting ~0.1 milliarcsecond (100 μas) for stellar surface imaging.

- With a ~500 m baseline (common in designs): ~230 μas at 550 nm visible, or sharper in UV (~60–120 μas at shorter wavelengths like 1550 Å).

- Some designs aim for baselines up to 1 km or more → down to ~50–100 μas.

**How much more can we see compared to Hubble?**

- Hubble: ~0.058 arcseconds (58,000 μas).

- Advanced space interferometer (e.g., 500 m–1 km baseline, UV/optical): ~50–200 μas.

- **Improvement factor**: Roughly **300–1,000 times finer angular resolution** than Hubble (often cited as 100–1000× in mission documents for stellar imaging).

This translates to seeing:

- **Sun-like stars at 4–10 parsecs (~13–30 light-years)**: ~700–1000 resolution elements across the disk (like mapping ~30–50 pixels across the equator), enough to resolve **starspots**, **plages**, active regions, granulation, flares, and magnetic structures—similar to how we image the Sun today but on other stars.

- **Giant/supergiant stars** (much larger angular sizes): Detailed surface maps, convection cells, mass ejections, or binary interactions.

- **Protoplanetary disks, accretion disks, jets**: Fine structures in forming planetary systems or around black holes.

- **Exoplanet host stars**: Potential to detect surface features influenced by planets or resolve close companions.

Compared to your 2 km baseline (~57 μas):

- A scaled-up array with similar or slightly shorter baselines but many more telescopes → similar or slightly coarser raw resolution per baseline, but **far more detail overall** because:

- Multiple baselines "fill" the imaging plane → full 2D reconstructed images (not just 1D profiles).

- Total light collection increases dramatically (e.g., 20 × 1 m mirrors ≈ collecting area of a ~4.5 m telescope) → better signal-to-noise, fainter details visible, shorter exposures, hyperspectral data.

- Result: You go from "resolving the star as a disk with basic features" to "high-fidelity maps with hundreds to thousands of resolution elements," revealing dynamic processes over time (e.g., rotation, cycles, seismology).

### Even Further Potential

- **Longer baselines** (10–20 km, feasible in future lunar or free-flying concepts): Down to ~5–10 μas or better → resolving white dwarfs, finer exoplanet atmosphere details, or transients on nearby stars.

- **Intensity interferometry** mode (robust for long baselines): Could push to km+ scales with simpler tech, enabling ~microarcsecond imaging on bright sources.

In short: Starting from your 2 km dual-telescope idea (~1000× Hubble), going to a full multi-telescope formation-flying array pushes to **several hundred to a thousand times sharper than Hubble**, enabling the first true "stellar surface imaging" revolution—mapping magnetic activity, habitability clues, and stellar evolution in unprecedented detail. This is exactly what missions like the Stellar Imager were designed to achieve before being deprioritized.

If you'd like calculations for a specific baseline, wavelength, or target star distance, just say!